Solution 
for Example II: Configuration Calculation (Self Test)

Pixel Calculation

- Color 1 (Beam): 1

- Color 2 (Wash): 20

- White: 15

- Pan: 1

- Tilt: 1

- Zoom: 20

Total configured Pixels: 58

Max allowed Pixels: 66

→Pixel Count within limit

Static Block Calculation

- Color 1 (Beam) (Individual): 3

- Color 1 (Beam) (Shared): 1 (Master Strobe)

- Color 2 (Wash) (Individual): 0

- Color 2 (Wash) (Shared):  0

- White (Individual): 0

- White (Shared):  1 (White Strobe)

- Pan: 1

- Pan Endless Rotation: 1

- Tilt: 1

- Tilt Endless Rotation: 1

- Zoom: 1

- Focus: 1

- Frost: 1

- Prisma: 1

- Iris: 1

Total configured Static Blocks: 14

Max allowed Static Blocks: 12

→ Static Blocks not within limit

FX Block Calculation

- Color 1 (Beam) (Individual): 8

- Color 1 (Beam) (Shared): 1 (Master Intensity)

- Color 2 (Wash) (Individual): 3

- Color 2 (Wash) (Shared):  0

- White (Individual): 1

- White (Shared):  0

- Pan: 1

- Pan Endless Rotation: 0

- Tilt: 1

- Tilt Endless Rotation: 0

- Zoom: 1

- Focus: 0

- Frost: 0

- Prisma: 0

- Iris: 0

Total configured FX Blocks: 16

Max allowed FX Blocks: 17

→ Static Blocks within limit

→ Result: Configuration is not valid for nRF52833. The fixture needs the nRF52840, since in the nRF52840 there can be configured up to 24 Static Blocks.